Section 2.3.

Diseases of fish

CHAPTER 2.3.0.

general information

. . .

B.  MATERIALS AND BIOLOGICAL PRODUCTS REQUIRED FOR THE 
ISOLATION AND IDENTIFICATION OF FISH PATHOGENS

. . .

2.	Techniques

. . .

2.5.	Use of molecular techniques for surveillance testing, confirmatory testing and diagnosis (third paragraph)

As with all PCR protocols, optimisation may be necessary depending on the reagents, equipment and the plasticware. PCR is prone to false-positive and false-negative results. False-positive results (negative samples giving a positive reaction), may arise from either product carryover from positive samples or, more commonly, from cross-contamination by PCR products from previous tests. Therefore, each assay and tissue extraction should include a negative control to rule out contamination. False-negative results (positive samples giving a negative result), may occur due to the presence of a new variant that is not recognised by the PCR primer/probe set, which may lead to unwanted transmission of pathogens and biosecurity failure.

[…]

___________________________

Return to Agenda