Annex 23

### USA COMMENTS IN BLUE FONT

### CHAPTER 7.5.

#### ANIMAL WELFARE DURING SLAUGHTER

[…]

Article 7.5.30.

Electrical water-bath stunning for poultry

- Animal welfare concerns

In electrical water-bath stunning poultry are inverted and shackled by the legs from a shackle line. The bird's head has direct contact with the water-bath, and an electric current is passed from the water through the bird to the leg shackle. Hazards that may prevent effective electrical stunning are: lack of contact between head and water, differences in individual bird resistance, improper system grounding, pre-stun shocks due to wings contacting water before the head, and the use of inappropriate electrical parameters (low voltage/current or high frequency).

Hazards that increase the likelihood of animals experiencing pre-stun shocks are: poor handling at shackling, inappropriate line speed, physical contact between birds, incorrect angle of entry ramp, entry ramp wetted by charged water, incorrect water-bath height, and shallow immersion.

Factors affecting individual bird resistance include the resistance between the shackle and the leg (leg/shackle interface), shackling on top of a severed foot, shackling by one leg, poor shackle position, incorrect shackle size, dry shackles, scale on the shackle surface, and keratinised skin on the legs (e.g. older birds).

Where insufficient inappropriate electrical stunning parameters are used, conscious animals are at risk of being electro-immobilised or paralysed causing pain and suffering.

- Animal-based and other measures

Multiple indicators should be used to determine whether a stun is effective and the animal is unconscious. Animal-based measures of an effective stun are: tonic-clonic seizures; apnoea; and absence of corneal or palpebral reflex.

Animal-based measures of ineffective stun or recovery of consciousness are: vocalisation; spontaneous blinking; righting reflex; presence of corneal or palpebral reflex; rhythmic breathing; spontaneous swallowing; and head shaking.breathing (identified by movements of the beak or abdominal muscles), wing flapping and making efforts to stand upright.

RATIONALE: Language in Point 2 is more applicable to livestock and should be edited to reflect poultry signs of regaining consciousness to align with this section concerning electrical water-bath stunning for poultry.

https://pmc.ncbi.nlm.nih.gov/articles/PMC9951727/

- Recommendations

The height of the water-bath stunner should be adjusted so that the birds' heads are completely immersed in the water. Avoid distractions such as people walking under the birds because this can cause birds to pull up.

Personnel should watch for short or stunted birds as these birds will not be able to make contact with the water and will not be stunned. These birds should be stunned in the slaughter line (e.g. penetrative captive bolt) or removed and euthanised.

RATIONALE: Recommend deleting stunning in the slaughter line language, as the referenced birds (i.e. short or stunted) would not realistically continue in the slaughter line.

The rail of the shackle line should run smoothly, use breast rubs and be dimly lit. Sudden movement such as jolts, drops or sharp curves in the line may cause birds to flap and avoid the stunner.

RATIONALE: Both appropriate lighting (i.e., dim lighting, blue lighting) to keep birds calm during and after shackling (pre-stun) and use of appropriate and well-maintained breast rubs from the shackling area to the entrance to the stunner keep birds calm and reduce wing movement, vocalization and stress prior to stunning.

https://www.avma.org/sites/default/files/2024-09/Humane-Slaughter-Guidelines-2024.pdf

To minimise any disturbance to birds during shackling, where shackles are wet to improve conductivity, they should be wetted only prior to birds' legs being placed in them.

Pre-stun shocks should be avoided and can be reduced by having a smooth shackle line and entry to the water-bath and by adjusting the water level of the bath to minimise overflow.

In the case of ineffective stunning or recovery, animals should be re-stunned using a backup system or be killed immediately. Ineffective stunning or return to consciousness should be systematically recorded and the cause of the failure identified and rectified.

Stunning equipment should be used, cleaned, maintained and stored following the manufacturer's recommendations.

Constant current stunners should be preferred to constant voltage stunners because the former ensure that the minimum current is provided to the animals independently from their impedance.

Regular calibration of the equipment according to the manufacturer's procedure is recommended. Effectiveness of the stunning should be monitored regularly.

Slaughterhouses/abattoirs should have standard operating procedures that define key operating parameters or follow the manufacturer's recommendations for stunning, such as:

  - water level;

  - salinity (salt concentration)

  - water temperature

RATIONALE: Recommend adding salinity and water temperature as important operating parameters.

https://www.avma.org/sites/default/files/2024-09/Humane-Slaughter-Guidelines-2024.pdf

  - number of birds in the water-bath;

  - contact between water and head, as well as between the legs and the leg shackle;

  - electrical parameters (current intensity [A], waveform type [AC and DC], voltage [V] and frequency [Hz]);

  - visual or auditory warning system to alert the operator to proper or improper function, such as a device that monitors and displays voltage and applied current.

Ensure an optimum combination of minimum current intensity, voltage and frequency (for DC) during electrical water-bath stunning practices, to maximise the effectiveness of stunning. Lower frequencies ensure an effective stunning. Higher frequencies provide the lowest probability of a successful stun even at the highest current intensity.

RATIONALE: Recommend the addition of “minimum current” to better describe “intensity” and clarify parameters are minimum values which may need to be increased, depending on the resistance of the animal. Misinterpretation of recommended values can lead to under stunning.

Hazards to animal welfare such as inversion of conscious birds, pre-stun shocks, and variability in electrical current delivered to each bird are inherent risks of electrical water-bath stunning. Thus, alternative stunning systems which avoid these associated hazards should be preferred.

- Species-specific recommendations

Effective electrical parameters should be based on scientific evidence for different species of birds.

Effective electrical parameters should be based on scientific data with field evidence on the welfare outcomes for different types and conditions of animals in accordance with point 5 of Article 7.1.4.

For water-bath stunning depending on the frequency, minimum recommended parametersminimum current intensities are recommended for the following species are:

RATIONALE: Recommend the addition of “minimum current” to better describe “intensity” and clarify parameters are minimum values which may need to be increased, depending on the resistance of the animal. Misinterpretation of recommended values can lead to under stunning.

  - For frequency below 200 Hz:

    - 100 mA for chicken,

    - 250 mA for turkeys,

    - 130 mA for ducks and geese,

    - 45 mA for quails.

  - For frequency from 200 to 400 Hz:

    - 150 mA for chicken,

    - 400 mA for turkeys.

  - For frequency from 400-600 Hz:

    - 200 mA for chicken,

    - 400 mA for turkeys.

Birds should receive the current for at least 4 seconds.

For dDucks, geese and quails should not be stunned at frequencies higher than 200 Hz will not achieve effective stunning and therefore are not recommended. [under study].

For cChicken and turkeys should not be stunned at frequencies higher than 600 Hz will not achieve effective stunning and therefore are not recommended. [under study].

[…]

_________________